SPECIFICATION TEMPLATE

RITEFRAME Steel Sub-frame System for Decking

- SCOPE OF WORK

The scope of work includes the design, supply, fabrication and installation RITEFRAME Steel Sub-frame System, complete with all necessary sub-structures, anchors, hardware and fittings to provide a total installation of the decking sub-frame system ready for decking installation.

- MATERIAL AND FINISHES

Decking Sub-frame Material:

Shall be RITEFRAME Steel Sub-frame System supplied by Blue Chip Group Pty Ltd (Ph: 08 9451 2344) cold-formed according to AS/NZS:4600 from Galvanised and/or Zincalume G300 steel which is continuous hot-dipped aluminium/zinc alloy-coated structural steel as per AS:1397.

**NO ALTERNATIVE MATERIALS WILL BE ACCEPTED FOR THIS PROJECT**

Sub-framing System General:

The sub-framing system to be installed in a manner to ensure all applied loadings to the decking is transferred back to the ground. Size and spacing of sub-frame members shall be determined according to applied loads and deflection limitations. Joist spacings shall be installed at maximum 450mm centres or as required to adequately support the decking system.

Fixings General:

Fasteners, including concealed screws, nuts, bolts and other items required for connecting aluminium to aluminium or aluminium to steel shall be in accordance with AS 3566.2 and of a type to suit its application and exposure conditions.

Class 1/2:	Internal applications.

Class 3:	External applications, moderate industrial and marine applications.

Class 4:	Severe marine applications

Dissimilar Materials:

Where two surfaces of dissimilar material come into contact, such surfaces shall be separated with a layer of PVC or Polyethylene tape or powder-coat finish.

Pedestals:

If required decking pedestals shall be RITEJACK adjustable pedestal system supplied by Blue Chip Group Pty Ltd and installed as required to meet or exceed project specific loading requirements and in accordance with the manufacturer’s instructions.

Disclaimer:

This Specification provided is generic in nature and is supplied in good faith for Specification guidance purposes only. It is understood that no 2 projects are alike and project specific information and advice should be obtained from necessary engineers, consultants and suppliers on a project to project basis.