SPECIFICATION TEMPLATE

RITESCREEN Through-colour Composite Screening

- SCOPE OF WORK

The scope of work includes the design, supply, fabrication and installation RITESCREEN Through-colour Composite Screening, complete with all necessary sub-structures, anchors, hardware and fittings to provide a total installation and screening system from the structure out.

- MATERIAL AND FINISHES

Screening Material:

The Screening shall be RITESCREEN Through-colour Composite Screening supplied by Blue Chip Group Pty Ltd (Ph: 08 9451 2344) consisting of an extra tough, commercial grade composite material made using a mixture of virgin and recycled plastic and recycled timber fibres. RITESCREEN wood composite Screening offers a highly practical combination of quality, durability and sustainability. It is highly suitable for external use in even the harshest of environments. Contributing to RITESCREEN's outstanding physical properties, is the use of patented technology, which offers superior structural strength and sustainability and increased UV resistance.

**NO ALTERNATIVE MATERIALS WILL BE ACCEPTED FOR THIS PROJECT**

Colour Selection:

Refer to exterior finishes schedule.

(Select colour code/s from the Finishes tab at the below link) http://www.bluechipgroup.net.au/timber-cladding-perth/composite-screening-perth.html

Batten Size Selection:

Refer to exterior finishes schedule.

(Select batten sizes from the Availability tab at the below link) http://www.bluechipgroup.net.au/timber-cladding-perth/composite-screening-perth.html

Space Between Battens:

Refer to exterior finishes schedule.

(Alternatively enter nominal space between battens in millimetres here: ______________ )

Fire Properties:

- Manufactured by Blue Chip Group Pty Ltd; RITECLAD Timber Composite Cladding is not non-combustible however it achieves the below fire performance indices when tested to AAS 1530.3 in accordance with current NCC/BCA requirements.

| RITESCREEN |  |  |  |
| --- | --- | --- | --- |
| TEST STANDARD | RESULT |  |  |
| BAL Rating | Up to BAL19 (By fire engineer evaluation) |  |  |
| AS1530.3 | PASS | Ignitability Index | 10 |
|  | PASS | Heat Evolved | 6 |
|  | PASS | Spread of Flame | 5 |
|  | PASS | Smoke Developed | 4 |

Fixings:

Fasteners, including concealed screws, nuts, bolts and other items required for connecting aluminium to aluminium or aluminium to steel shall be in accordance with AS 3566.2 and of a type to suit its application and exposure conditions.

Class 1/2:	Internal applications.

Class 3:	External applications, moderate industrial and marine applications.

Class 4:	Severe marine applications

- INSTALLATION

The Screening system shall be installed by an approved Screening subcontractor with a demonstrated experience of at least 5 years in the fabrication and installation of pre-finished Screening systems. All work to be carried out in accordance with the RITESCREEN manufacturer's recommendations and installation manuals. All component parts shall be installed level, true to line with uniform joints and reveals.

Installation System Code:

RS-24-VSF: Visible Screw Fix (PDF & DWG construction details available on request)

- SPAN TABLES

Non-load Bearing Vertical Applications*

| Supports Per Length of Product | Maximum Recommended Centres |
| --- | --- |
| 2 | 450mm |
| 3 | 600mm |
| 4 | 900mm |
| 5+ | 1200mm |

*Vertical applications refer to wall cladding, fence screening etc

Non-load Bearing Horizontal Applications**

| Supports Per Length of Product | Maximum Recommended Centres |
| --- | --- |
| 2 | 450mm |
| 3 | 600mm |
| 4 | 900mm (Screening only) |
| 5+ | 1200mm (Screening only) |

**Horizontal applications refer to decking, seating, ceiling lining, patio screening etc

Load Bearing Horizontal Applications**

| Supports Per Length of Product | Maximum Recommended Centres |
| --- | --- |
| 2 | 300mm |
| 3+ | 450mm |

**Horizontal applications refer to decking, seating, ceiling lining, patio screening etc

Disclaimer:

This Specification provided is generic in nature and is supplied in good faith for Specification guidance purposes only. It is understood that no 2 projects are alike and project specific information and advice should be obtained from necessary engineers, consultants and suppliers on a project to project basis. The above span tables do not remove the responsibility from the client to seek an engineer’s approval for all load bearing or structural applications and is intended solely as a guide for ensuring good aesthetic performance of the products. It is not possible to encompass all different applications, so it is strictly for informational purposes and does not constitute a warranty in any way shape or form.