SPECIFICATION TEMPLATE

TROPIKA Stain-proof Composite Decking

- SCOPE OF WORK

The scope of work includes the design, supply, fabrication and installation TROPIKA Stain-proof Composite Decking, complete with all necessary sub-structures, anchors, hardware and fittings to provide a total installation and decking system from the structure out.

- MATERIAL AND FINISHES

Decking Material:

The decking shall be TROPIKA Stain-proof Composite Decking supplied by Blue Chip Group Pty Ltd (Ph: 08 9451 2344) consisting of an extra tough, commercial grade composite material made using a mixture of virgin and recycled plastic and recycled timber fibres. TROPIKA wood composite decking offers a highly practical combination of quality, durability and sustainability. It is highly suitable for external use in even the harshest of environments. Contributing to TROPIKA's outstanding physical properties, is the use of patented technology, which offers superior structural strength and sustainability and increased UV resistance.

**NO ALTERNATIVE MATERIALS WILL BE ACCEPTED FOR THIS PROJECT**

Colour Selection:

Refer to exterior finishes schedule.

(Select colour code/s from the Finishes tab at the below link) http://www.bluechipgroup.net.au/timber-decking-perth/stain-proof-composite-decking-perth.html

Profile Selections:

Deck Boards	TROPIKA 140 x 22mm Secret Fix Decking **Insert Colour**, 5.4m Lengths

Edge Boards	TROPIKA 140 x 22mm Pencil Round **Insert Colour**, 5.4m Lengths

Secret Fixings:

The TROPIKA secret fix decking boards shall be installed using the proprietary STOWAWAY secret-fixing clips with black-head coated stainless steel square-drive screws installed in accordance with the manufacturer’s instructions.

Face Fixings:

The TROPIKA edge boards, fascia boards and decking breaker boards as required shall be fixed using the proprietary COLOURFIX colour-matched composite decking screws installed in accordance with the manufacturer’s instructions.

Fixings General:

Fasteners, including concealed screws, nuts, bolts and other items required for connecting aluminium to aluminium or aluminium to steel shall be in accordance with AS 3566.2 and of a type to suit its application and exposure conditions.

Class 1/2:	Internal applications.

Class 3:	External applications, moderate industrial and marine applications.

Class 4:	Severe marine applications

Dissimilar Materials:

Where two surfaces of dissimilar material come into contact, such surfaces shall be separated with a layer of PVC or Polyethylene tape or powder-coat finish.

Warranty:

The decking shall be covered by a 25 year manufacturer’s warranty. All work to be carried out in accordance with the manufacturer's recommendations and installation details. The warranty is subject to the decking system being fabricated and installed by a manufacturer trained and approved installer with a minimum 5 years’ experience.

- INSTALLATION

The decking system shall be installed by an approved decking subcontractor with a demonstrated experience of at least 5 years in the fabrication and installation of pre-finished decking systems. All work to be carried out in accordance with the TROPIKA manufacturer's recommendations and installation manuals. All component parts shall be installed level, true to line with uniform joints and reveals.

Sub-framing System:

The sub-framing system to be installed in a manner to ensure all applied loadings to the decking is transferred back to the ground. Size and spacing of sub-frame members shall be determined according to applied loads and deflection limitations. Joist spacings shall be installed at maximum 450mm centres or as required to adequately support the decking system.

Acceptable Sub-framing materials are:

- RITEFRAME Steel Sub-framing System

- RITEJACK Decking Pedestals if required

- Suitably treated structural grade timber to suit the project location & requirements

- Available from Blue Chip Group Pty Ltd (Ph. 08 9451 2344), sales@bluechipgroup.net.au.

Steel Sub-frame Standards:

RITEFRAME steel sub-framing shall be cold-formed according to AS/NZS:4600 from Galvanised and/or Zincalume G300 steel which is continuous hot-dipped aluminium/zinc alloy-coated structural steel as per AS:1397.

Pedestals:

If required decking pedestals shall be RITEJACK adjustable pedestal system supplied by Blue Chip Group Pty Ltd and installed as required to meet or exceed project specific loading requirements and in accordance with the manufacturer’s instructions.

Installation System Code:

RD-24-CSF: Concealed Screw Fix (PDF & DWG construction details available on request)

Disclaimer:

This Specification provided is generic in nature and is supplied in good faith for Specification guidance purposes only. It is understood that no 2 projects are alike and project specific information and advice should be obtained from necessary engineers, consultants and suppliers on a project to project basis.